FREQUENTLY ASKED QUESTIONS

(GEOTECHNICAL ENGINEERING)

DEPARTMENT OF CIVIL ENGINEEIRNG

GIET UNIVERSITY, GUNUPUR

Geotechnical Engineering

Frequently Asked Questions

- What is soil?
Ans: Natural aggregate of mineral particles with water and air.

- What is soil mechanics?
Ans: Study of engineering behavior of soil.

- What is phase diagram of soil?
Ans: Representation of soil as solid, water, and air.

- What is void ratio?
Ans: Ratio of volume of voids to volume of solids.

- Define porosity.
Ans: Ratio of volume of voids to total volume.

- What is water content?
Ans: Ratio of weight of water to weight of solids.

- What is degree of saturation?
Ans: Percentage of voids filled with water.

- What is specific gravity of soil solids?
Ans: Ratio of unit weight of soil solids to water.

- What is dry unit weight?
Ans: Weight of dry soil per unit volume.

- What is bulk unit weight?
Ans: Total weight of soil per unit volume.

- What is saturated unit weight?
Ans: Unit weight when all voids are filled with water.

- What is submerged unit weight?
Ans: Difference between saturated and water unit weight.

- What is Atterberg limits?
Ans: Limits defining consistency of fine soils.

- What is liquid limit?
Ans: Water content at transition from liquid to plastic state.

- What is plastic limit?
Ans: Water content at transition from plastic to semi-solid state.

- What is shrinkage limit?
Ans: Water content below which no volume change occurs.

- What is plasticity index?
Ans: Difference between LL and PL.

- What is liquidity index?
Ans: Measure of soil consistency in natural state.

- What is consistency index?
Ans: Relative firmness of soil.

- What is particle size analysis?
Ans: Determination of soil grain distribution.

- What is sieve analysis?
Ans: Particle size analysis for coarse soils.

- What is hydrometer analysis?
Ans: Particle size analysis for fine soils.

- What is uniformity coefficient?
Ans: Measure of particle size range.

- What is coefficient of curvature?
Ans: Indicates gradation shape.

- What is well graded soil?
Ans: Soil with wide range of particle sizes.

- What is poorly graded soil?
Ans: Soil with narrow size range.

- What is IS classification of soil?
Ans: Indian Standard soil classification system.

- What is coarse grained soil?
Ans: Soil with >50% particles >75 micron.

- What is fine grained soil?
Ans: Soil with >50% particles <75 micron.

- What is clay?
Ans: Fine soil with plasticity.

- What is silt?
Ans: Fine soil with low plasticity.

- What is sand?
Ans: Coarse grained soil.

- What is gravel?
Ans: Coarse particles larger than sand.

- What is organic soil?
Ans: Soil containing organic matter.

- What is relative density?
Ans: Measure of compactness of sand.

- What is sensitivity of clay?
Ans: Ratio of undisturbed to remolded strength.

- What is activity of clay?
Ans: Ratio of PI to clay fraction.

- What is soil structure?
Ans: Arrangement of soil particles.

- What is flocculated structure?
Ans: Edge-to-face particle arrangement.

- What is dispersed structure?
Ans: Face-to-face particle arrangement.

- What is permeability?
Ans: Ability of soil to transmit water.

- What is Darcy’s law?
Ans: Flow rate proportional to hydraulic gradient.

- Define coefficient of permeability.
Ans: Velocity of flow under unit hydraulic gradient.

- What is hydraulic gradient?
Ans: Head loss per unit length.

- What is laminar flow?
Ans: Smooth orderly flow of water.

- What is turbulent flow?
Ans: Irregular flow of water.

- What is seepage velocity?
Ans: Actual velocity of water through voids.

- What is discharge velocity?
Ans: Flow rate per unit cross section.

- What is quick sand condition?
Ans: Soil loses strength due to upward seepage.

- What is critical hydraulic gradient?
Ans: Gradient at which soil becomes weightless.

- What is flownet?
Ans: Graphical representation of seepage.

- What is flow line?
Ans: Path followed by water particle.

- What is equipotential line?
Ans: Line of equal head.

- What is seepage force?
Ans: Force exerted by flowing water on soil.

- What is permeability test?
Ans: Lab test to determine permeability.

- What is constant head test?
Ans: Test for coarse grained soils.

- What is falling head test?
Ans: Test for fine grained soils.

- What factors affect permeability?
Ans: Grain size, void ratio, viscosity.

- What is anisotropic soil?
Ans: Permeability differs in directions.

- What is isotropic soil?
Ans: Permeability same in all directions.

- What is seepage pressure?
Ans: Pressure exerted by flowing water.

- What is uplift pressure?
Ans: Upward pressure due to seepage.

- What is piping?
Ans: Removal of soil due to seepage.

- What is exit gradient?
Ans: Hydraulic gradient at downstream end.

- What is pervious soil?
Ans: Soil allowing water flow easily.

- What is impervious soil?
Ans: Soil restricting water flow.

- What is capillary rise?
Ans: Upward movement of water in soil pores.

- What is seepage analysis?
Ans: Study of water flow through soil.

- What is flownet application?
Ans: Calculate seepage quantity.

- What is head loss?
Ans: Loss of energy in water flow.

- What is permeability of clay?
Ans: Very low.

- What is permeability of sand?
Ans: High.

- What is effective porosity?
Ans: Void space contributing to flow.

- What is hydraulic conductivity?
Ans: Same as permeability.

- What is seepage quantity?
Ans: Volume of water flowing per unit time.

- What is seepage force direction?
Ans: In direction of flow.

- What is artesian condition?
Ans: Pressurized groundwater.

- What is perched water table?
Ans: Localized groundwater above main table.

- What is permeability in situ test?
Ans: Field test of permeability.

- What is steady flow?
Ans: Flow constant with time.

- What is compaction?
Ans: Densification by mechanical means.

- What is consolidation?
Ans: Volume reduction due to expulsion of water.

- What is OMC?
Ans: Optimum moisture content.

- What is MDD?
Ans: Maximum dry density.

- What is Proctor test?
Ans: Lab compaction test.

- What is standard Proctor test?
Ans: Compaction with 2.5 kg rammer.

- What is modified Proctor test?
Ans: Higher energy compaction.

- What is zero air voids line?
Ans: Theoretical max density line.

- What is degree of compaction?
Ans: Ratio of field to lab dry density.

- What is field compaction test?
Ans: Sand replacement method.

- What is core cutter method?
Ans: Field density test.

- What is one-dimensional consolidation?
Ans: Settlement under vertical load.

- What is primary consolidation?
Ans: Settlement due to pore water drainage.

- What is secondary consolidation?
Ans: Creep settlement.

- What is compression index?
Ans: Slope of e-log p curve.

- What is recompression index?
Ans: Slope during unloading-reloading.

- What is coefficient of consolidation?
Ans: Rate of consolidation parameter.

- What is preconsolidation pressure?
Ans: Max past effective stress.

- What is normally consolidated soil?
Ans: Current stress equals max past stress.

- What is over consolidated soil?
Ans: Past stress > present stress.

- What is oedometer test?
Ans: Lab consolidation test.

- What is drainage path?
Ans: Distance water travels to drain.

- What is time factor?
Ans: Parameter for consolidation rate.

- What is degree of consolidation?
Ans: Fraction of settlement completed.

- What is coefficient of volume compressibility?
Ans: Volume change per unit pressure.

- What is settlement?
Ans: Downward movement of soil.

- What is immediate settlement?
Ans: Elastic settlement.

- What is Terzaghi’s theory?
Ans: One-dimensional consolidation theory.

- What is load increment ratio?
Ans: Ratio of successive loads.

- What is consolidation curve?
Ans: Plot of void ratio vs pressure.

- What is compaction curve?
Ans: Plot of dry density vs water content.

- What is kneading compaction?
Ans: Compaction by rolling and pressing.

- What is vibratory compaction?
Ans: Compaction by vibration.

- What is impact compaction?
Ans: Compaction by ramming.

- What is compaction energy?
Ans: Energy applied during compaction.

- What is field compaction control?
Ans: Checking field density.

- What is swelling?
Ans: Volume increase on wetting.

- What is shrinkage?
Ans: Volume decrease on drying.

- What is consolidation settlement formula?
Ans: S = (Cc H / (1+e₀)) log(σf/σ0)

- What is drainage condition?
Ans: Single or double drainage.

- What is total stress?
Ans: Stress due to soil + water weight.

- What is pore water pressure?
Ans: Pressure of water in pores.

- What is effective stress?
Ans: Stress carried by soil skeleton.

- State effective stress principle.
Ans: σ' = σ – u

- Who proposed effective stress theory?
Ans: Terzaghi.

- What is neutral stress?
Ans: Pore water pressure.

- What is buoyant unit weight?
Ans: γsat – γw

- What is submerged soil?
Ans: Soil below water table.

- What is capillary stress?
Ans: Negative pore pressure above water table.

- What is seepage pressure effect?
Ans: Alters effective stress.

- What happens to σ' during upward seepage?
Ans: Decreases.

- What happens to σ' during downward seepage?
Ans: Increases.

- What is quick condition?
Ans: Zero effective stress.

- What is effective stress at ground surface?
Ans: Zero.

- What is overburden pressure?
Ans: Stress due to overlying soil.

- What is saturated soil condition?
Ans: All voids filled with water.

- What is partially saturated soil?
Ans: Voids filled with air + water.

- What is pore pressure parameter?
Ans: Ratio of excess pore pressure to applied stress.

- What is excess pore pressure?
Ans: Pressure due to loading.

- What is drained condition?
Ans: Pore water allowed to escape.

- What is undrained condition?
Ans: No pore water escape.

- What is effective overburden pressure?
Ans: Vertical effective stress.

- What is stress distribution?
Ans: Spread of load in soil.

- What is Boussinesq theory?
Ans: Stress under point load.

- What is Westergaard theory?
Ans: Stress with layered soil.

- What is influence factor?
Ans: Factor for stress calculation.

- What is pressure bulb?
Ans: Zone of stress influence.

- What is contact pressure?
Ans: Pressure at footing base.

- What is bearing stress?
Ans: Stress under foundation.

- What is uplift due to pore pressure?
Ans: Reduction in effective stress.

- What is effective stress path?
Ans: Stress change path.

- What is stress history?
Ans: Past loading condition.

- What is initial effective stress?
Ans: Stress before loading.

- What is final effective stress?
Ans: Stress after consolidation.

- What is pore pressure dissipation?
Ans: Reduction over time.

- What is consolidation pressure?
Ans: Increase in effective stress.

- What is critical state?
Ans: Soil shears at constant volume.

- What is stress anisotropy?
Ans: Unequal stresses in directions.

- What is isotropic stress?
Ans: Equal stress in all directions.

- What is total vertical stress formula?
Ans: σ = γ × z

- What is shear strength of soil?
Ans: Resistance to shear deformation.

- What is Mohr-Coulomb theory?
Ans: τ = c + σ' tanφ

- What is cohesion?
Ans: Shear strength at zero normal stress.

- What is angle of internal friction?
Ans: Resistance due to particle interlocking.

- What is shear stress?
Ans: Tangential stress on soil plane.

- What is normal stress?
Ans: Stress perpendicular to plane.

- What is drained shear strength?
Ans: Strength with drainage allowed.

- What is undrained shear strength?
Ans: Strength without drainage.

- What is triaxial test?
Ans: Shear strength test under 3D stress.

- What is UU test?
Ans: Unconsolidated undrained test.

- What is CU test?
Ans: Consolidated undrained test.

- What is CD test?
Ans: Consolidated drained test.

- What is direct shear test?
Ans: Simple shear strength test.

- What is vane shear test?
Ans: Field test for soft clays.

- What is failure envelope?
Ans: Locus of shear failure points.

- What is peak shear strength?
Ans: Maximum shear resistance.

- What is residual shear strength?
Ans: Strength after large deformation.

- What is critical state shear strength?
Ans: Strength at constant volume.

- What is dilatancy?
Ans: Volume increase on shearing.

- What is strain hardening?
Ans: Strength increase with strain.

- What is strain softening?
Ans: Strength decrease after peak.

- What is effective cohesion?
Ans: Cohesion in effective stress terms.

- What is apparent cohesion?
Ans: Cohesion due to suction.

- What is sensitivity of clay?
Ans: Strength loss on remolding.

- What is failure plane?
Ans: Plane along which soil fails.

- What is principal stress?
Ans: Maximum and minimum normal stress.

- What is Mohr circle?
Ans: Graphical stress representation.

- What is confining pressure?
Ans: Lateral pressure in triaxial test.

- What is shear modulus?
Ans: Ratio of shear stress to strain.

- What is factor of safety in shear?
Ans: Ratio of available to mobilized strength.

- What is bearing capacity failure?
Ans: Shear failure under footing.

- What is general shear failure?
Ans: Sudden failure with bulging.

- What is local shear failure?
Ans: Progressive failure.

- What is punching shear failure?
Ans: Footing punches into soil.

- What is mobilized shear strength?
Ans: Strength at working load.

- What is critical shear plane?
Ans: Plane of max shear stress.

- What is shear band?
Ans: Localized deformation zone.

- What is stress path in shear?
Ans: Variation of stress during loading.

- What is cohesionless soil?
Ans: Soil with c ≈ 0.

- What is cohesive soil?
Ans: Soil with significant cohesion.