FREQUENTLY ASKED QUESTIONS

(STRUCTURAL ANALYSIS I)

DEPARTMENT OF CIVIL ENGINEEIRNG

GIET UNIVERSITY, GUNUPUR

STRUCTURAL ANALYSIS I

Frequently Asked Questions

- What is a statically determinate structure?
A structure solvable using equilibrium equations alone.

- What is a statically indeterminate structure?
A structure requiring compatibility conditions in addition to equilibrium.

- How many equilibrium equations exist in 2D?
Three.

- How many equilibrium equations exist in 3D?
Six.

- What is degree of static indeterminacy?
Excess unknown reactions over equilibrium equations.

- What is a kinematically determinate structure?
A structure with no possible rigid body movement.

- What is kinematic indeterminacy?
Number of independent joint displacements.

- What is a perfect frame?
A stable and statically determinate frame.

- Condition for perfect pin-jointed truss?
m = 2j − 3.

- What is a redundant member?
A member causing indeterminacy.

- Define stable structure.
A structure resisting displacement under load.

- Define unstable structure.
A structure incapable of maintaining equilibrium.

- Example of statically determinate beam?
Simply supported beam.

- Example of indeterminate beam?
Fixed beam.

- What is external indeterminacy?
Indeterminacy due to excess reactions.

- What is internal indeterminacy?
Indeterminacy due to excess members.

- What is total indeterminacy?
Sum of internal and external indeterminacy.

- What causes kinematic instability?
Insufficient restraints.

- What is a rigid joint?
Joint preventing relative rotation.

- What is a hinge joint?
Joint allowing rotation but no moment transfer.

- Define degree of freedom.
Independent displacement parameter.

- What is a plane structure?
Structure with forces in one plane.

- What is a space structure?
Structure with forces in three dimensions.

- What is a redundant reaction?
Reaction exceeding equilibrium requirements.

- What is compatibility condition?
Displacement consistency requirement.

- What is a stable determinate structure called?
Perfect structure.

- What is a mechanism?
Structure undergoing motion without resistance.

- Define constraint.
Restriction preventing movement.

- What is a fixed support restraint?
Restricts translation and rotation.

- What is a roller support restraint?
Restricts vertical translation only.

- What is a pin support restraint?
Restricts horizontal and vertical translation.

- What is partial restraint?
Incomplete restriction of motion.

- What is structural redundancy?
Presence of extra load paths.

- Why indeterminate structures preferred?
Better load redistribution.

- Disadvantage of indeterminate structures?
Complex analysis.

- What is static analysis?
Analysis using equilibrium equations.

- What is kinematic analysis?
Analysis based on displacements.

- What is a continuous beam?
Beam with more than two supports.

- Is a cantilever determinate?
Yes.

- Is a propped cantilever indeterminate?
Yes.

- What is an influence line diagram (ILD)?
Graph showing variation of response due to moving load.

- Who introduced influence lines?
Müller-Breslau.

- ILD is mainly used for?
Moving load analysis.

- ILD for reaction is linear or curved?
Linear.

- ILD for bending moment is?
Curved.

- Maximum BM occurs when?
Load is at section.

- ILD for cantilever reaction is?
Constant.

- ILD for simply supported beam reaction?
Triangular.

- ILD for shear force is?
Discontinuous at section.

- ILD for BM is zero at supports?
Yes.

- What load gives maximum shear?
Load near the section.

- What load gives maximum BM?
Load at section.

- What is absolute maximum BM?
Maximum BM anywhere in beam.

- ILD for overhanging beam?
Piecewise linear.

- ILD ordinate represents?
Response per unit load.

- What is moving load?
Load changing position.

- ILD for truss member force is?
Piecewise linear.

- Method to draw ILD theoretically?
Müller-Breslau principle.

- ILD used in bridge design?
Yes.

- ILD depends on magnitude of load?
No.

- ILD depends on geometry?
Yes.

- ILD for fixed beam?
Not applicable directly.

- Maximum SF occurs when?
Load is just adjacent to section.

- ILD ordinate sign indicates?
Nature of response.

- ILD for BM at mid-span is?
Parabolic.

- What is a wheel load?
Concentrated moving load.

- What is UDL on ILD?
Area under ILD × load intensity.

- ILD for pin-jointed truss reaction?
Linear.

- ILD for axial force is?
Straight line segments.

- ILD for cantilever BM at fixed end?
Straight line.

- ILD helps in?
Critical load positioning.

- ILD for shear has jump due to?
Concentrated load.

- ILD is independent of material?
Yes.

- ILD maximum ordinate indicates?
Critical section.

- ILD used for railway bridges?
Yes.

- ILD for support moment?
Negative region exists.

- ILD shape governed by?
Structural form.

- ILD for symmetric beam is?
Symmetric.

- ILD drawn for unit load?
Yes.

- ILD for truss depends on?
Load path.

- What is a three-hinged arch?
Arch with hinges at supports and crown.

- Is three-hinged arch determinate?
Yes.

- Normal thrust acts along?
Axis of arch.

- Shear force acts?
Perpendicular to arch axis.

- BM at hinges is?
Zero.

- Parabolic arch best suited for?
UDL.

- Segmental arch shape is?
Circular.

- Horizontal thrust depends on?
Span and rise.

- Three-hinged arch is insensitive to?
Support settlement.

- ILD for arch thrust is?
Curved.

- Maximum thrust occurs when?
Load at crown.

- BM in three-hinged arch under UDL is?
Zero.

- What is rise of arch?
Vertical distance between crown and springing.

- What is span of arch?
Horizontal distance between supports.

- Arch resists load by?
Compression.

- Suspension bridge cable resists?
Tension.

- Cable shape under UDL?
Parabolic.

- Cable shape under point loads?
Polygonal.

- Stiffening girder function?
Distributes load.

- Horizontal tension in cable is?
Constant.

- Three-hinged stiffening girder is?
Statically determinate.

- BM in cable is?
Zero.

- Maximum sag occurs at?
Mid-span.

- ILD for cable tension is?
Linear.

- Suspension bridges suitable for?
Long spans.

- Anchor block resists?
Cable tension.

- Hangers transfer load by?
Tension.

- Arch thrust reduces with?
Increased rise.

- Arch under point load develops?
Bending moment.

- Three-hinged arch BM at crown is?
Zero.

- Arch is compression structure?
Yes.

- Cable is tension structure?
Yes.

- Suspension bridge flexibility is?
High.

- Arch flexibility is?
Low.

- ILD for shear in stiffening girder?
Discontinuous.

- ILD used in arch analysis?
Yes.

- Temperature effect on three-hinged arch?
Negligible.

- Arch analysis uses?
Equilibrium.

- Suspension bridge analysis uses?
Compatibility.

- BM in stiffening girder is?
Non-zero.

- What is superposition principle?
Total response equals sum of individual responses.

- Valid for linear systems only?
Yes.

- Maxwell’s reciprocal theorem states?
Deflection at A due to load at B equals vice-versa.

- Betti’s law relates?
Two load systems work equivalence.

- Strain energy stored due to?
Deformation.

- Strain energy unit?
Joule.

- Virtual work principle applies to?
Equilibrium systems.

- Unit load method used for?
Deflection calculation.

- Castigliano’s theorem relates?
Strain energy and displacement.

- Partial derivative of strain energy gives?
Deflection.

- Strain energy in axial member equals?
∫(P²/2AE)dx.

- Strain energy due to bending equals?
∫(M²/2EI)dx.

- Virtual displacement is?
Imaginary infinitesimal displacement.

- Real work equals virtual work when?
System is in equilibrium.

- Unit load applied where?
At point of desired displacement.

- Energy methods best suited for?
Indeterminate structures.

- Virtual work method accuracy is?
High.

- Energy methods require material linearity?
Yes.

- Deflection depends on?
Load, geometry, stiffness.

- Castigliano’s theorem applicable to?
Elastic systems.

- First theorem gives?
Deflection.

- Second theorem gives?
Redundant forces.

- Energy stored due to shear often?
Neglected.

- Energy stored due to torsion equals?
∫(T²/2GJ)dx.

- Unit load is applied in direction of?
Desired displacement.

- Virtual work method uses?
Internal and external work equality.

- Energy methods are?
General methods.

- Compatibility ensured by?
Energy minimization.

- Strain energy is always?
Positive.

- Energy methods simplify?
Complex analysis.

- Double integration method based on?
Beam differential equation.

- Macaulay’s method simplifies?
Discontinuous loading.

- Moment-area method uses?
Area of M/EI diagram.

- Conjugate beam method transforms?
Deflection problem into shear problem.

- Slope equals?
First derivative of deflection.

- Deflection equals?
Second integral of curvature.

- Maximum deflection occurs where?
Slope is zero.

- Boundary condition at fixed end?
Zero slope and deflection.

- Boundary condition at simple support?
Zero deflection.

- Column is member under?
Compression.

- Strut carries?
Axial compression.

- Short column failure mode?
Crushing.

- Long column failure mode?
Buckling.

- Euler’s theory applicable to?
Long columns.

- Euler critical load proportional to?
EI/L².

- Effective length depends on?
End conditions.

- Effective length for pinned-pinned?
L.

- Effective length for fixed-fixed?
L/2.

- Effective length for fixed-free?
2L.

- Euler theory assumes?
Perfect straightness.

- Euler formula ignores?
Material yielding.

- Slenderness ratio equals?
Le/r.

- Buckling is?
Sudden lateral deflection.

- Column stiffness depends on?
EI.

- Moment-area theorem first gives?
Change in slope.

- Moment-area theorem second gives?
Deflection.

- Conjugate beam support transformation?
Fixed becomes free.

- Macaulay’s method uses?
Bracket notation.

- Deflection proportional to?
Load magnitude.

- Deflection inversely proportional to?
EI.

- Columns designed against?
Buckling.

- Euler load independent of?
Material strength.

- Long column stress is?
Low.

- Short column stress is?
High.

- Critical load means?
Buckling load.

- Elastic stability theory applies to?
Columns.

- End restraint increases?
Buckling load.

- Effective length factor for pinned-fixed?
0.7L.

- Deflection analysis required for?
Serviceability.

- Slope measured in?
Radians.

- Why study structural analysis?
To predict structural behavior.

- Primary aim of analysis?
Safety and economy.

- Structural analysis precedes?
Design.

- Linear analysis assumes?
Small deformations.

- Elastic range obeys?
Hooke’s law.

- Why deflection control is important?
Serviceability.

- Energy methods best for?
Complex structures.

- ILD critical in?
Bridge engineering.

- Columns mainly resist?
Axial compression.

- Structural stability ensures?
Safe load transfer.