|  | GIET UNIVERSITY, GUNUPUR – 765022 B. Tech (Fourth Semester – Regular) Examinations, April / May – 2021 Sub. Code – Sub. Name (Agricultural Engineering) |
| --- | --- |
| Time: 3 hrs Maximum: 70 Marks |  |

Answer ALL Questions-1(Q&A)

The figures in the right hand margin indicate marks.

--- End of Paper ---

Answer ALL Questions-1(Q&A)

The figures in the right hand margin indicate marks.

PART – A: (Multiple Choice Questions)                                                               (1 x 10 = 10 Marks)

Q.1. Answer ALL questions

a.	In cantilever beam, slope and deflection at free end is _____

(ii)  Maximum.

b.	Which of the following statements is/are true for a simply supported beam?

(i) Deflection is maximum at a point where slope is zero.

c.	The vertical distance between the axis of the beam before and after loading at a point is called as _______

(iii) Deflection.

d.	The load at which a vertical compression member just buckles is known as

(iv)  Any one of these.

e.	A column that fails due to direct stress is called

(i) Short column.

f.	Riveted joint is ...... joints

(ii) Permanent.

g.	A masonry dam may fail due to

(iv) Any one of the above

h.	Portion of dam in contact with ground at downstream side is __________

(ii)  Heel.

i.	In cantilever beams, the extra support is known as ____________

(i)  Prop.

j.	Fixed beam is also known as _______

(iii)  Encaster beam.

PART – B: (Short Answer Questions)                                                                             (2 x 10 = 20 Marks)

Q.2. Answer ALL questions

a.	What is slope of beam?

The deviation of point B to B' is shown as deflection δB and the change in slope of tangent at B is shown as slope θB

b.	What are the methods for finding out the slope and deflection at a section?

Double integration method, Moment–area method, Mecaulay’s method, Conjugate beam method.

c.	Why moment area method is more useful, when compared with double integration?

Moment area method is more useful, as compared with double integration method because many problems which do not have a simple mathematical solution can be simplified by the ending moment area method.

d.	Define the strut.

A structural member, subjected to an axial compressive force, is called as strut.

e.	What are the types of end conditions of column?

Both ends are hinged or pinned, One end is free and the other is fixed,Both ends are fixed.

One end is fixed and the other end is pinned.

f.	List types of welded joints.

Butt joint, Tee joint, Corner joint, Lap joint, Edge joint.

g.	What is masonry dam?

Masonry dams are dams made out of masonry – mainly stone and brick, sometimes joined with mortar. They are either the gravity or the arch-gravity type

h.	Write the stability of dam.

Stability and stress analyses are the most important elements that require rigorous consideration in design of a dam structure. Stability of dams against sliding is crucial due to the substantial horizontal load that requires sufficient and safe resistance to develop by mobilization of adequate shearing forces along the base of the dam foundation.

I          What do you understand by the term ‘prop’?

A beam having one built-in support and one simple support. For ex consider one end of the beam supported by fixed support and the other end by roller support such beams can be called as propped cantilever beam.

j.	What is meant by on encastre beam?

A beam is a structural element that primarily resists loads applied laterally to the beam's axis. ...     Fixed or Encastre – a beam supported on both ends and restrained from rotation.

PART – C: (Long Answer Questions)                                                                             (10 x 4 = 40 Marks)

Answer ALL questions

3. a.	Derive the relation between slope, deflection and radius of curvature of a simply supported beam.

\

A small portion PQ of the beam, bent into an arc is considered .

Let ds be the elemental length PQ of the beam

R be the radius of the arc

C be the centre of the arc (into which the beam has been bent)

ψ be the angle which the tangent at P makes with x − x axis and

(ψ + dψ) be the angle which the tangent at Q makes with x − x axis

From the geometry of Figure 8.3,

∠PCQ = dψ and

ds = R. dψ

R = dsdψ = dxdψ

1R = dψdx

We know that if x and y be the co- ordinates of point P, then

tanψ = dydx

Differentiating the above equation with respect to x

dψdx = d2 ydx2

We know

MI = ER  OR      M = EIR

M = EId2ydx2

b.	A timber beam of rectangular section has a span of 4.8 m and is simply supported at its ends. it is required to carry a total load of4500 kg uniformly distributed load over the whole span.Find the maximum value for (a) the breadth and  depth of the beam, if the maximum bending stress is not exceed 70 kg /cm2 and the maximum deflection is limited to 9.5 mm. take E  for timber as 105 x 103 kg/cm2

Given data

Span (l) = 4.8 m

Uniformly distributed load (W) = wl = 4500 kg

Bending stress (f) =70 kg/cm2

Maximum deflection (yc) = 9.5 mm = 0.95 cm

Young’s modulus (E) = 105 x 103 kg/cm2

Solution

We know

M =WL8 = 4500 x 4.88 = 2700 kg – m

We know

MI = fy,		2700,000bd312 = 70d2

bd3 = 270,000 x 1270 x 2 =23 143 cm2

yc = 5Wl3384 EI = 5 x 4500 x 480 3384 x 105 x103 x bd312 =5184 0007bd3

bd3  = 5 184 0007 x 0.95 = 779 500 cm2

d = 779 50023 143 = 33.68 cm,	b x 33.683 = 23 143, b = 23 14333.683 = 20.4 cm

4. a.	An I section joist 40 cm x 20 cm x 2 cm and 6 m long is used as a strut with both end fixed. What is Euler’s crippling load for the column take Young’s modulus for the joist as 2.0 x 106 kg/ cm2

Given data

(b) = 20 cm

Outer depth (d) = 40 cm

Inner width (b1) = 20 – 2 = 18 cm

Inner depth (d1) = 40 – 2 x 2 36 cm

Length (l) = 6 m = 600 cm

Young’s modulus (E) = 2.0 x 106 kg/cm2

Solution

We know that moment of inertia about X –X  axis

Ixx = 112bd3 - b1d13 = 112 20 x 40 3- 18 x 36 3 = 36 680 cm4

Iyy  = 2 x 20312 + 36 x 2312 = 1357.3 cm4

Equivalent length (L) =l2 = 6002 = 300 cm

P = π2 x EIL2 = π2 x 2 x 106 x 1357.33002 = 297700 kg

(OR)

b.	A single riveted lap joint is made is 15 mm thick plates with 20 mm diameter rivets. Determine the strength of the joint, if the pitch of the rivet is 6 cm. Take fs = 900 kg/cm2, fb = 1600 kg/cm2, and ft = 1200 kg/cm2

Given data

Plate thickness (t) = 15 mm = 1.5 cm

Diameter of rivets (d) = 20 mm = 2 cm

Pitch (p) = 6 cm

Shearing stress (fs) = 900 kg/ cm2

Bearing stress (fb) = 1600 kg/ cm2

Tearing stress (ft) = 1200 kg/ cm2

Solution

We know, shearing of rivets

Ps = fs x π4 x d2 = 900 x π4 x22  = 2827 kg

Bearing of rivets

Pb = fb x t x d =1600 x 1.5 x 2 = 4800 kg

Tearing of plate

Pt = ft x p-d x t = 1200 (6 – 2) x1.5 = 7200 kg

5. a.	A tank is filled with a liquid of sp.gr. 1 up to a depth of 0.5 m over another liquid of sp.gr. 2. Find total pressure per metres length and its line of action, on the I m deep size of the tank

Given data

Specific gr. Of upper liqiud = 1

Specific weight (w1) = 1000 kg/m3

Depth of upper liquid (h1) = 0.5 m

Specific gr. Of lower liqiud = 2

Specific weight (w2) = 2000 kg/m3

Depth of upper liquid (h2) = 0.5 m

Solution

We know,

P1 = Total pressure of upper liquid per metre length ,

P2 = Total pressure of lower liquid per metre length

We know that the pressure BD = w1h1= 1000 x 0.5 = 500 kg m2

Pressure EF = w2h2 = 2000 x 0.5 = 1000 kg/m2

Total pressure of upper liquid per metre length

P1 = Area of triangle ABD x length of wall = (12 X 500 X 0.5 ) x 1 = 125 kg

Total pressure of lower liquid per metre length

P2  = Area of figure BDFC x length of wall = (A rea of rectangle BCDE + Area of triangle DEF) x length of wall

= (500 x  0.5 ) x  + (12 x 1000 x 0.5) x1 = 500 kg

Total pressure = 125 + 500 = 625 kg

H = depth of the  line of action of the total pressure from A

Taking moments all pressure about A

P x h = pressure ABD x 2 x 0.53 + Pressure BCDE x 0.5+ 0.52 x Pressure DEF x 0.5+ 2 x 0.53

625h = 125 x13 + 250 x 34 + 250 x 56

h = 26256 x 625 = 0.7 m

(OR)

b.	A masonry trapezoidal dam 4 m high 1 m wide at its top and 3 m wide its bottom retains water on its vertical face. Determine the maximum and minimum stresses, at the base,(i) when the reservoir  is full and (ii) when the reservoir is empty. Take weight of the masonry as 2000 kg/m3

Given data

Height of dam (H) = 4 m

Top width (a) = 1 m

Bottom width (b) = 3 m

Weight of masonry (ρ) = 2000 kg/ cm3

Solution

Let

Pmax = maximum stress at the base B

Pmin = minimum stress at the base A

Stress at the base when the reservoir is full.

Let 	x = horizontal distance between c.g of the dam section and K

P = Total pressure of water per metre length

Using the relation

P = wh32 = 1000 x 432 = 8000 kg

The weight of the dam per metre length, W = ρ x a+b2  x H = 2000 x 1+32 x 4  = 16000 kg

Find the position, taking moments of area about A

4 x 1+4 x 22 AJ = 4 x 1 x12 + 4 x 22 1+ 23

8AJ =2 + 6.67 = 8.67, AJ = 1.08 m

Using the relation

x = PW x h3 = 800016000 x 43 = 0.67 m

If horizontal AK, d = AJ + x = 1.08 + 0.67 = 1.75 m

Eccentricity, e = d - b2 = 1.75 - 32 = 0.25 m

Using the relation

Pmax =  Wb 1+6eb  = 160003 1+  6 x 0.253 = 8000 kg/m2

Pmin =  Wb 1- 6eb = 160003 1- 6 x 0.253  = 2667 kg/m2

Stress at the base when the reservoir is empty.

Using the relation

We know that the distance AJ, d = 1.08 m, and eccentricity, e = d- b2 = 1.08 -1.5 = -0.42 m

Using the relation

Pmax = Wb 1+ 6eb = 160003 1+ 6 x0.423 = 9813 kg/m2

Using the relation

Pmin =  W/b (1- 6e/b) = 16000/3  (1- (6 x 0.42)/3)  = 853 kg/m2

6. a.	A propped cantilever 10 m long has 15 cm wide and 40 cm deep cross section. If the allowable bending stress and the deflection at the centre is 100 kg/cm2 and 1.5 cm respectively, determine the safe uniformly distributed load, which cantilever can carry, take  E = 1.2 X 106 kg/cm2.

Given data

Span (l) = 10 m = 1000 cm

Width (b) = 15 cm

Depth (d) = 40 cm

Solution

Moment of inertia, I = bd312 =15 x 40312 =80000 cm4

Allowable bending stress at the centre (f) =100 kg/cm2

Allowable deflection at the centre (yc) 1.5 cm

Young’s modulus (E) =1.2 X 106  kg/cm2

Maximum bending moment (M) = wl38  =w x 100038 =125000 w kg-cm

Using the relation, 	MI = fy ,		125000 xw80000 = 10020	, 	w = 3.2 kg/cm

Using another the relation

yc = w l4192 EI , 	1.5 = W X 1000492 X 1.2 X106  X 8000 = 0.542 w

w = 2.765 kg/cm

(OR)

b.	A fixed beam AB of 5 m span carries a point load of 20 kN  at a distance of  2 m from A. Determine the values of fixing moments and deflection under the load, if flexural rigidity of the beam is 10 x 103 kN – m2

Given data

Span (l) = 5 m

Load (W) = 20 kN

Solution

Distance of load from A, a = 2 m

Distance of load from B, b = 5 - 2 m = 3 m

Flexural rigidity, (EI) = 10 x 103 kN – m2

Fixing moments

Let 	MA = Fixing moment at A and

MB = Fixing moment at B

Using the relation

MA = -Wab2l2 = - 20 x 2 x3252 =- 14.4 kN –m

Now using the relation

MB = Wa2 bl2 = 20 x 22 x 352 = - 9.6 kN –m2

Let y = deflection under the load

Using the relation

Y = Wa3 b33L3 EI = 20 x 23 x 333 x 53 x 10 x 103 = 0.0015 m